# Maritime Hackathon 2026

# Emissions Calculation Methodology

Refer below procedure and data in the vessel_movement dataset for calculations.

## Step 1: Determine the operating mode of the vessel at each AIS data point

GPS speed provides movement activity which can be used to categorize vessels as either at anchorage, maneuvering, in transit, or drifting.

There are three operating modes:

- Anchorage: in_anchorage = 1 AND speed_knots <1.

- Maneuver: in_port_boundary = 1 AND speed_knots >1.

- Transit: in_port_boundary = 0 AND speed_knots >=1.

- Vessel movements that do not fall under the three operating modes outlined above can be considered as 'Drifting'.

## Step 2: Log event activity hours at each timestamp

The event activity hour (A) is the difference between two consecutive timestamps for each vessel i.e. same vessel id.

## Step 3: Calculate Vessel movement parameters

### Step 3a: Calculate maximum speed (MS)

Maximum Speed (MS) = 1.066 x Vref

### Step 3b: Calculate Engine Load Factor (LF) for Main engine

LF=(AS/MS)3
Where
LF = Load Factor
AS = Actual ship speed (knots) from AIS vessel_movements data set
MS = Maximum speed (knots)

- Do standard rounding the LF to two decimal places.

- If LF < 0.02 and mode is transit or maneuver, then LF should be defaulted to 0.02.

Refer Calculation factor document from this point onwards.

### Step 4: Fuel consumption calculations

### Step 4a: Calculate each ship’s adjusted specific fuel consumption (sfc_adjusted_xy) for each machinery

The columns sfc_xy provided in dataset corresponds to a default Distillate fuel type. This step makes sfc specific to the fuel the ship’s machineries will actually be burning.

sfcadjustedxy=sfcxy*(42.7LCV)

Where:

xy = Main_engine(me), Aux_engine(ae), Aux_Boiler(abl)

HINT:

- Refer columns main_engine_fuel_type, aux_engine_fuel_type, and aux_boiler_fuel_type from vessel_movement_dataset and find corresponding LCV from the Cf table sheet to use in above formula.

### Step 4b: Machinery Fuel consumption (tonnes)

For:

Main Engine = LF*P*sfcadjustedme*A1000000

Auxiliary Engine =ael*sfcadjustedae*A1000000

Auxiliary Boiler = abl*sfcadjustedblr*A1000000

### Step 5: Emission Calculations

### Step 5a: Look up Low Load Adjustment Factor (LLAF) table using LF calculated in Step 3b

- When referring the LLAF table provided, the LF calculated in step 3b should be multiplied by 100 to get a percent number (%LF).

- Do standard arithmetic rounding of the %LF to nearest integer before looking up LLAF Table.

- LLAF will be 1 if %LF is more than 20%.

- If %LF < 2% and mode is transit or maneuver, then LF will be defaulted to 2% when looking up the LLAF table.

### Step 5b: Emissions from machineries

Refer Cf table provided.

For each of the three machineries stated earlier, calculate the three greenhouse gas emissions:

- Emission_pqr (tonnes) = LLAFpqr*Cfpqr*fuel consumptionxy

Where:

pqr = CO2;   N2O;   CH4  and

xy = Main_engine(me), Aux_engine(ae), Aux_Boiler(abl)

- Total Emissions of each gas (pqr) = Sum of Emission_pqr from each machinery

Example: Total Emission_pqr = Main Engine_pqr + Aux Engine_pqr + Aux Boiler_pqr

- Total Equivalent CO2 emissions, CO2eq = ∑(GWPpqr*Total Emissionpqr)

Where:

GWP = 1, 265, 28 for CO2, N2O and CH4 respectively.

Note: Fuel and related emissions only for  transit and maneuvering modes are to be calculated for within the scope of the hackathon.

### Step 6: Cost Calculations

Step 6a: Fuel Cost - Refer Fuel cost table

Cost per tonne of fuel = Cost per GigaJoule (GJ) * LCV of main_engine_fuel_type

Total fuel cost (USD) = Total fuel consumed by all machineries * Cost per tonne of fuel

Step 6b: Emission Cost - Refer Cost of Carbon table

Total Carbon cost (USD) = Total Equivalent CO2 emissions * Carbon cost per ton (2024)

Step 6c: Ship Ownership Cost - Refer Cost of ship table.

- Asset depreciation rate (r) = 8% per annum

- Life of each ship (N) = 30 years

- Cost of each ship =

Distillate fuel ship cost as per DWT * Multiplier factor (M) as per main_engine_fuel_type

HINT:	40-55k DWT means:		 40,000 < DWT <=55,000

- Salvage Cost i.e. End of life value = 10% of cost of ship

- Capital Recovery Factor (crf) = {r*(1+r)^N}/{((1+r)^N)-1}

- Amortized annual ownership cost of ship (million USD) = ((P-S)*CRF)+(r*S)

HINT: The annual cost is to be converted to monthly cost as cargo in scope is 1-month. Read Problem statement document.

Step 6d: Total cost of ship (monthly)

Total cost of ship (monthly) = Fuel cost + Emission cost + Ship ownership cost

Step 6e: Risk Premium (Safety Score adjustment)

Refer Safety Score Adjustment table.

As per the Safety score of each ship, the risk premium will get applied as follows:

Risk Premium (USD) = Total Cost of ship (monthly) * adjustment rate

Take note of the +/- notation provided in the table.

Step 6f: Final risk adjusted ship cost

Adjusted cost (USD) = Total monthly cost of ship +/- Risk Premium

Final answer should be in USD.

Step 7: Fleet fuel consumption, emissions, and cost

All below values are for the final fleet selected as per problem statement

Total fuel consumption = Sum of fuel consumed by each selected ship

Total cost of fleet = Sum of adjusted cost of each selected ship

Total emissions of CO2 equivalent = Sum of CO2 equivalent emissions of each ship